Gesture-Controlled Robot Arm

Project Review

Dr Oisin Cawley   |   April 2026

Student Information

Name: Jake Powell Langstaff

Student ID: C00287662

Course: Bachelor of Science (Honours) in Cybercrime and IT Security

Github: https://github.com/JakePowellLangstaff/robot_arm_code-

YouTube: https://www.youtube.com/shorts/da9gX9PGN7k

Abstract

This project built a gesture-controlled robotic arm using a standard laptop webcam, Google's MediaPipe hand-tracking library, and a five-joint Lynxmotion Smart Servo arm. The system detects hand gestures in real time and translates them into pre-programmed movement sequences executed over a serial connection. Two gestures drive the core demo: raising one finger triggers a full pickup sequence, and a peace sign runs the drop-off and return. A third gesture, an open palm, resets the arm to its starting position at any point.

The project involved writing a series of test scripts to map out each servo's physical range, resolving a library compatibility issue with MediaPipe's updated API, and iterating on movement timing until the arm operated smoothly enough to be reliable in demonstration conditions. The final system runs as a single Python file and requires no keyboard input during operation.

Introduction

Robotic arms have traditionally been controlled through physical interfaces such as joysticks, control panels, or pre-written scripts. This project explored a different approach: using real-time hand gesture detection through a webcam to trigger and control arm movements, with no physical controller involved.

The hardware used was a Lynxmotion Smart Servo arm with five independently controlled joints covering base rotation, shoulder, elbow, wrist, and a gripper claw. The software side combined Python, OpenCV for camera capture, and Google's MediaPipe framework for hand landmark detection. Serial communication via the LSS library handled the connection between the code and the physical servos.

The goal was a system that could reliably pick up an object from one fixed location and deposit it at another, driven entirely by gestures shown to the camera. The review that follows covers what was built, the problems encountered along the way, and what the project produced.

1. The Process

The aim was to control a robotic arm using hand gestures picked up through a standard laptop webcam. Detect what shape the hand is making, translate it into a command, send the command to the arm. Getting there involved considerably more back-and-forth than expected, and several things that should have been straightforward turned out not to be.

Getting the arm working

The arm has five Lynxmotion Smart Servo joints, each addressable over a serial connection on COM7. Before any gesture recognition was involved, the first task was just understanding what each joint could actually do. A series of small test scripts were written to move each servo through its range one at a time and record where it hit its physical limits. This turned out to be more important than it sounds, because several of the assumed ranges were wrong.

LSS2 and LSS3 (shoulder and elbow) caused the most confusion early on. The first arm used negative values to move upward. When a second arm was introduced mid-project, this turned out to be completely reversed. Positive values went up, and sending negative values either did nothing or triggered a fault. Every movement sequence written up to that point had to be redone from scratch.

LSS5 (the claw) had a different kind of range problem. The assumed safe open position was 800, but commanding the servo there caused it to go red — the fault indicator. After isolating the claw in a dedicated test script and stepping through values slowly, the confirmed safe open position turned out to be -600. Any further than that and the servo faulted every time.

The MediaPipe compatibility problem

Getting hand tracking running hit an unexpected wall early on. The original code used the older MediaPipe interface:

mp_hands = mp.solutions.hands

This worked in the documentation examples, but the only version available in the project environment was MediaPipe 0.10.33, which had removed the solutions namespace entirely. Downgrading was not an option because no older build existed for the Python version being used. The detection layer had to be rewritten using the newer Tasks API, which requires downloading a separate model file rather than bundling the model into the library. The landmark data the new API returns is identical — the same 21 coordinate points per hand per frame — so the gesture classification logic required no changes at all. It was a frustrating blocker that turned out to have a clean solution once the root cause was properly understood.

Building the movement sequences

With the arm and camera both working independently, the job was combining them. Sending a servo directly to its target position in one command technically worked but looked terrible — joints snapped to position with no sense of controlled motion. A helper function was written to break any move into equal increments:

def smooth_move(name, servo, start, goal, steps=3):

The step count and pause between steps could be adjusted per joint. The base rotation covers a large range and needed ten or more steps to avoid a jarring sweep. The wrist could manage with four or five. This function was used in the standalone position test scripts first, and the resulting waypoints were then expanded individually into the gesture-controlled sequences.

One decision that changed late in the process was removing automatic return-to-home at the end of the pickup sequence. Early versions sent the arm back to start after every grab, but it made the demo feel rushed and removed the purpose of the home gesture. Keeping the arm holding its position after a pickup, and letting the drop-off sequence handle the full return, made the overall flow much more coherent.

The home gesture also changed. A closed fist was originally mapped to home, but the camera regularly confused it with a relaxed or slightly open hand. Switching to an open palm made detection noticeably more reliable. The shape is visually distinct enough that it is much harder to trigger accidentally.

2. The Result

The final system runs as a single Python file. On startup, the arm moves to a fixed starting position — base rotated to one side, all other joints at neutral, claw open — and holds there while the camera reads for input.

One finger triggers the pickup sequence. The wrist angles down first, then the elbow drops, then the shoulder rotates in two stages to bring the arm into position over the item. The claw closes in four steps rather than one snap, giving it time to settle around the object. The arm holds the item and waits.

A peace sign runs the drop-off and the return as a single uninterrupted sequence. The arm adjusts the wrist, sweeps the base to the drop-off location, opens the claw to release the item, and then without any further input lifts the shoulder first to clear the workspace, returns the elbow and wrist to neutral, and rotates back to the start. By the time the sequence ends, the arm is back exactly where it started, claw open, ready for the next pickup.

Open palm sends all joints home immediately at any point — an emergency reset available throughout the entire runtime.

The camera window displays which gesture is being detected in real time, and the status label changes colour while a sequence is running so it is always clear whether the arm is waiting or busy.

3. The Learnings

The most useful thing this project reinforced was that physical testing cannot be skipped. A lot of the early assumptions about servo ranges were simply wrong, and no amount of reading documentation would have caught any of them. The claw faulting at 800, the second arm using reversed sign conventions, the wrist angle being off — all of those were discovered by running the code and watching the hardware. Writing short, isolated test scripts for individual joints before combining anything saved considerable time and made faults much easier to trace.

Threading had to be added once movement sequences were integrated with the camera. Without it, the camera window froze for the entire duration of any arm movement — sometimes fifteen seconds or more — which made the system feel broken even when everything was working. Running sequences in a background thread while the main loop kept reading camera frames was a small change that made the whole system feel responsive.

The MediaPipe version issue was a useful reminder not to assume library stability between major versions. The fix required reading and understanding the new API properly rather than patching around the error, which was the right call. Once the rewrite was done properly, nothing had to be revisited.

Switching from fist to open palm for the home gesture was a practical lesson in the difference between what works conceptually and what works in front of a camera. A fist is a clear concept but a visually ambiguous shape at an angle, in varied lighting, or with a relaxed hand. An open palm has a much cleaner landmark profile and is harder to trigger accidentally. Gesture design has to hold up under the conditions the camera will actually face, not just on paper.

The step-based movement approach made a visible difference to how the project read as a demonstration. A joint snapping instantly to its target looks like something malfunctioning. Breaking each move into smaller steps with controlled pauses made the arm look deliberate and controlled, which matters when the point is to show that gestures are driving something physical.

Fine-tuning through physical testing

Some adjustments only became possible once an actual object was in front of the arm. The wrist position during pickup was initially set to -600, which put the claw at a slightly low approach angle — arriving from below rather than straight on. Raising it to -700 corrected the angle enough to make gripping consistent. It is exactly the kind of thing that cannot be anticipated from coordinate values alone.

The choice of test object also mattered. An energy drink can was used first, but the round cylindrical surface gave the claw very little reliable contact area. Switching to a whiteboard eraser gave the gripper a flatter, more predictable shape to work with, and the pickup became noticeably more consistent as a result.

Wait times between movement steps needed more iteration than expected. The original value of 1.2 seconds per step made sequences painfully slow. Cutting to 0.4 seconds introduced stutter — servos were being commanded before finishing the previous position. After testing, 0.9 seconds was settled on for most joints. The base rotation (LSS1) was the exception: because it sweeps with the arm tucked close in and carries less collision risk, the timing could be pushed tighter without losing control.

Conclusion

The project ended up in a very different place from where it started. The initial idea was straightforward, but the combination of hardware surprises, library version breaks, and the gap between simulated and real physical behaviour meant that almost every component had to be rethought at least once. The system that came out of it does what it was supposed to do — picks up an object on one gesture, drops it off on another, handles the return journey automatically, and does it smoothly enough that the gestures read as the cause rather than an afterthought.

What stayed consistent throughout was the approach: test small, test often, and do not assume that anything carries over from one configuration to the next without being checked. That turned out to be the most transferable lesson of the whole project.

Appendix — Final Source Code

The following is the complete source code for the gesture-controlled arm system (gesture_arm_lssv3.py). Minor condensing applied to fit the page; full file available on GitHub.

###############################################################################

# Gesture-Controlled Robot Arm — LSS1-LSS5

# Dr Oisin Cawley  |  April 2026

# MediaPipe 0.10+ (Tasks API)

#

# GESTURES:

#   Open palm  (all fingers up)  ->  HOME  (return to starting position)

#   One finger (index only)      ->  PICKUP_SEQUENCE_1  (pick up item)

#   Peace sign (index+middle)    ->  PICKUP_SEQUENCE_2  (drop off + return)

#

# CONFIRMED JOINT RANGES:

#   LSS1 base    :  0=straight | -=right      | +=left

#   LSS2 shoulder:  0=down     | +=up (1300=max)

#   LSS3 elbow   :  0=parallel | -=down (-900=lowest)

#   LSS4 wrist   :  0=neutral  | -=down (-800=lowest)

#   LSS5 claw    :  -600=open  | 0=closed

###############################################################################

import os, time, urllib.request, threading

import cv2

import mediapipe as mp

from mediapipe.tasks import python as mp_python

from mediapipe.tasks.python import vision as mp_vision

import lss

import lss_const as lssc

CST_LSS_Port = "COM7"

CST_LSS_Baud = lssc.LSS_DefaultBaud

CAMERA_INDEX = 0

CONFIDENCE   = 0.75

MODEL_URL  = "https://storage.googleapis.com/mediapipe-models/...hand_landmarker.task"

MODEL_PATH = "hand_landmarker.task"

PICKUP_SEQUENCE_1 = [

# LSS4 wrist:  0 to -700  (6 steps)

{"lss4": -120, "wait": 0.6}, {"lss4": -240, "wait": 0.6},

{"lss4": -360, "wait": 0.6}, {"lss4": -480, "wait": 0.6},

{"lss4": -600, "wait": 0.6}, {"lss4": -700, "wait": 0.6},

# LSS3 elbow:  0 to -900  (5 steps)

{"lss3": -180, "wait": 0.9}, {"lss3": -360, "wait": 0.9},

{"lss3": -540, "wait": 0.9}, {"lss3": -720, "wait": 0.9},

{"lss3": -900, "wait": 0.9},

# LSS2 shoulder stage 1:  0 to 900

{"lss2":  180, "wait": 0.9}, {"lss2":  360, "wait": 0.9},

{"lss2":  540, "wait": 0.9}, {"lss2":  720, "wait": 0.9},

{"lss2":  900, "wait": 0.9},

# LSS2 shoulder stage 2:  900 to 1300

{"lss2":  980, "wait": 0.9}, {"lss2": 1060, "wait": 0.9},

{"lss2": 1140, "wait": 0.9}, {"lss2": 1220, "wait": 0.9},

{"lss2": 1300, "wait": 0.9},

# LSS5 claw close:  -600 to 0

{"lss5": -450, "wait": 0.9}, {"lss5": -300, "wait": 0.9},

{"lss5": -150, "wait": 0.9}, {"lss5":   -5, "wait": 0.9},

]

PICKUP_SEQUENCE_2 = [

# wrist adjust, base sweep, claw open, then full return

{"lss4": -800, "wait": 0.9},

{"lss1":  900, "wait": 1.2},   # base rotates across

{"lss4": -600, "wait": 0.9},

{"lss5": -600, "wait": 0.9},   # claw opens — item released

# shoulder up, elbow back, wrist back, base returns

{"lss2":    0, "wait": 0.9},

{"lss3":    0, "wait": 0.9},

{"lss4":    0, "wait": 0.9},

{"lss1": -900, "wait": 0.9},   # back to start

]

HOME = {"lss1": -900, "lss2": 0, "lss3": 0, "lss4": 0, "lss5": -600}

class SequenceRunner:

def __init__(self):

self._running = False

self._thread  = None

@property

def busy(self): return self._running

def run(self, sequence, servo_map, label):

if self._running: return

self._thread = threading.Thread(

target=self._execute, args=(sequence, servo_map, label), daemon=True)

self._thread.start()

def _execute(self, sequence, servo_map, label):

self._running = True

for step in sequence:

wait  = step.get('wait', 0.9)

moves = {k: v for k, v in step.items() if k != 'wait'}

for joint, pos in moves.items():

if joint in servo_map: servo_map[joint].move(pos)

time.sleep(wait)

self._running = False

def _finger_up(lm, tip, mcp):

return lm[tip].y < lm[mcp].y

def classify(lm):

idx   = _finger_up(lm,  8,  5)

mid   = _finger_up(lm, 12,  9)

ring  = _finger_up(lm, 16, 13)

pinky = _finger_up(lm, 20, 17)

up    = sum([idx, mid, ring, pinky])

if up == 4:              return 'OPEN'

if up == 1 and idx:      return 'ONE_FINGER'

if up == 2 and idx and mid: return 'PEACE'

return 'UNKNOWN'

def main():

lss.initBus(CST_LSS_Port, CST_LSS_Baud)

myLSS1=lss.LSS(1); myLSS2=lss.LSS(2); myLSS3=lss.LSS(3)

myLSS4=lss.LSS(4); myLSS5=lss.LSS(5)

servo_map = {'lss1':myLSS1,'lss2':myLSS2,'lss3':myLSS3,

'lss4':myLSS4,'lss5':myLSS5}

def go_home():

for joint, pos in HOME.items(): servo_map[joint].move(pos)

runner = SequenceRunner()

go_home(); time.sleep(3.0)

detector = mp_vision.HandLandmarker.create_from_options(...)

cap = cv2.VideoCapture(CAMERA_INDEX)

last_gesture = 'UNKNOWN'

while True:

ret, frame = cap.read()

frame = cv2.flip(frame, 1)

result = detector.detect(mp.Image(...))

if result.hand_landmarks:

gesture = classify(result.hand_landmarks[0])

if gesture != last_gesture:

if gesture == 'OPEN' and not runner.busy:

go_home()

elif gesture == 'ONE_FINGER' and not runner.busy:

runner.run(PICKUP_SEQUENCE_1, servo_map, 'PICKUP')

elif gesture == 'PEACE' and not runner.busy:

runner.run(PICKUP_SEQUENCE_2, servo_map, 'DROPOFF')

last_gesture = gesture

cv2.imshow('LSS Gesture Arm', frame)

if cv2.waitKey(1) & 0xFF == ord('q'): break

if __name__ == '__main__':

main()

Jake Powell Langstaff   |   C00287662   |   April 2026